RustyNet Blind-Exit PCB Report

Single-purpose low-level ARM exit-node PCB, firmware, and software readiness plan

Date: 2026-05-10 (Europe/Dublin)

| Field | Value |
| --- | --- |
| Assessment type | Static source review plus design report. No Rust build or test run was performed in this container. |
| RustyNet source archive | rustynet.zip ¦ SHA-256 56dced6e391c353b83a080ccf491f8d3a1eaa8d7eb7f035586be81c0892e7298 |
| PCB source archive | pcb.zip ¦ SHA-256 67fcad61097fea75069a4e33c7b3cd0acad7627d3a7b1f140a0b2b54c80a86b4 |
| Git commit of build | Unavailable: uploaded archives do not contain .git metadata, so git rev-parse HEAD cannot be recovered from the supplied files. |
| Build status in this environment | Not built: cargo and rustc were not installed in the execution container. The report is based on repository inspection. |
| Rust toolchain requested by repo | rust-toolchain.toml pins channel 1.88.0 with rustfmt and clippy. |

Commit caveat: The RustyNet release-gate documentation requires evidence reports to contain an exact 40-character git_commit matching git rev-parse HEAD. Because the uploaded zips contain no .git directory, this report cannot honestly provide a commit SHA. The archive hashes above are recorded as reproducibility anchors, not as a substitute for a git commit in release gates.

Evidence: FreshInstallOSMatrixReleaseGate.md:17-25 requires git_commit in evidence reports; filesystem inspection of /mnt/data/work/rustynet_zip/rustynet and /mnt/data/work/pcb_zip/pcb found no .git directory. Cargo.toml:1-17 lists the workspace members; rust-toolchain.toml:1-4 pins Rust 1.88.0.

# 1. Executive summary

Product definition. The target device is a sealed RustyNet blind-exit appliance: it receives RustyNet tunnel traffic from authorized mesh peers and routes that traffic to the wider internet via Ethernet or Wi-Fi uplink. It is not a relay node, not a general router, and not a user-modifiable Linux computer. The only intended user interaction is enrollment into a RustyNet mesh and, when required, uplink Wi-Fi provisioning.

Current software reality. RustyNet is not starting from zero. The repository already contains a blind_exit role, Linux WireGuard backend modes, daemon-side role restrictions, signed assignment/trust state, systemd units, key custody hardening, fail-closed controls, and nft/ip/sysctl helper validation. The core dataplane shape needed for an exit node is substantially present.

Current appliance gap. The repository still looks like host software operated through start.sh, roles, menus, and systemd installation. A dedicated appliance enrollment flow was not found. For this PCB, that general operator surface must be replaced by a one-time claim flow and a sealed runtime state machine.

Current PCB reality. The PCB archive describes an H616 prototype board originally scoped like a Pi Zero 2 W-class watchdog/UI appliance: 512 MB DDR3/DDR3L, 8 GB eMMC, microSD recovery, 10/100 Ethernet, integrated Wi-Fi, two USB-A ports, SPI TFT, and eight buttons. For a blind-exit PCB, that baseline needs to be simplified and redirected: remove the UI and general expansion affordances, improve networking and memory headroom, and make the board a sealed network appliance.

Recommendation. Use Debian/Armbian only for ARM prototype validation. The final device should use a small Linux image with a read-only root filesystem, narrow writable state, no package manager, no SSH by default, no permanent web UI, signed A/B updates, a physical factory-reset path, and a RustyNet runtime forced to blind_exit only.

Evidence: README.md:294-303 defines blind_exit as least-knowledge, immutable after setup, exit-serving, and unable to select an upstream exit. daemon.rs:930-1004 restricts blind_exit commands and route sanitization. rustyjack-h616-pcb-architecture-v0.md:10-23 and 47-58 define the existing H616/512 MB/10/100/UI/USB baseline. README.md:235-270 shows the current start.sh wizard/operator model.

# 2. Scope and non-goals

## 2.1 Product scope

- Primary function: act as a RustyNet exit node that forwards authorized mesh traffic to the public internet.

- Concurrency target from stakeholder discussion: normally about 3 concurrent users; maximum expected concurrent users about 8.

- Deployment model: plugged into power and either Ethernet or Wi-Fi uplink; left running unattended.

- Configuration model: claimable, not configurable. The device should only accept enough input to join a RustyNet mesh and optionally obtain Wi-Fi uplink credentials.

- Runtime identity: blind_exit only. The device must not be convertible into admin, client, relay, or general-purpose peer mode without factory reset/reprovisioning.

Authorized RustyNet peers

|

v

RustyNet / WireGuard tunnel interface (rustynet0)

|

v

Linux forwarding + NAT + fail-closed firewall policy

|

v

WAN uplink: eth0 preferred, wlan0 optional

|

v

Wider internet

Source classification: The 3/8-user concurrency target and the sealed-product requirement come from the stakeholder discussion, not from the current repos. Repository-derived claims are explicitly backed by source file references throughout this report.

## 2.2 Explicit non-goals for the blind-exit PCB

- No relay-node role on this device. Relay infrastructure may still be needed elsewhere in the RustyNet network for plug-and-play reachability, but this PCB is not the relay.

- No permanent dashboard, router web UI, SSH service, package manager, exposed shell, compiler, or generic Linux user surface.

- No display, eight-button UI, HDMI, exposed GPIO header, or user-facing USB expansion for the product revision.

- No LAN-access-by-default. RustyNet users should receive internet egress only unless a separately signed policy explicitly enables LAN access.

- No promise that the device is physically tamper-proof. The practical goal is appliance-locked and tamper-resistant within the chosen cost class.

Evidence: README.md:297 and daemon.rs:966-972 support blind_exit least-privilege behavior. Requirements.md:80-89 defines LAN access as an explicit toggle/route-policy concept and requires traffic/DNS fail-close behavior.

# 3. Build and source state

| Item | Observed state | Implication |
| --- | --- | --- |
| RustyNet git commit | Unavailable; no .git metadata in uploaded rustynet.zip. | Cannot satisfy release-gate requirement for exact git_commit from this archive alone. |
| PCB git commit | Unavailable; no .git metadata in uploaded pcb.zip. | PCB report can reference file content and archive hash, but not a source-control commit. |
| Rust build | Not executed; cargo and rustc absent in this container. | Software status is static inspection, not fresh build/test evidence. |
| Rust toolchain | Repo pins Rust 1.88.0, minimal profile, rustfmt and clippy. | Actual build environment should install exactly this toolchain before generating release evidence. |
| Workspace shape | Workspace includes rustynetd, rustynet-cli, rustynet-relay, backend-api, backend-wireguard, backend-stub, backend-userspace, control, policy, crypto, DNS, sysinfo, and Windows-native crates. | The software has already been split into daemon, CLI, relay, control, and backend surfaces; the appliance can reuse those libraries but should not ship the full operator surface. |

Evidence: Cargo.toml:1-17 lists workspace members; Cargo.toml:20-30 sets edition/version/license/authors and forbids unsafe_code workspace-wide; rust-toolchain.toml:1-4 pins toolchain 1.88.0. FreshInstallOSMatrixReleaseGate.md:17-25 specifies the required measured report fields, including git_commit.

Reproducibility anchors: RustyNet tree manifest SHA-256: 19e52f2b246cf1d0ab8d6c41745300a73c9068612e5e44def966894d0df774a6. PCB tree manifest SHA-256: 02a5fd461166355d244b141f5cb83db49de5a3766979f366238cb4227a7f087c. These are generated from sorted per-file hashes of the extracted archives and are useful for confirming the exact reviewed files, but they do not replace git commit identity.

# 4. Repository-confirmed RustyNet software state

## 4.1 Blind-exit role behavior

- Role exists: NodeRole includes Admin, Client, and BlindExit; the parser accepts blind_exit and blind-exit.

- Least-privilege command set: blind_exit accepts only status, netcheck, state refresh, and DNS inspect commands. It rejects exit-node selection, exit off, LAN toggles, route advertisement, key rotation, and other local mutation commands.

- Final-hop posture: README states blind_exit remains final-hop oriented: it can serve exit but cannot select an upstream exit.

- Exit-serving enforced: daemon bootstrap treats blind_exit as serve_exit_node=true while keeping its own exit_mode off. This matches a node that serves egress for others rather than tunneling its own traffic upstream.

- Client-only assignment sanitized: daemon clears selected_exit_node, disables LAN access, and ignores selected_exit_node/LAN assignment fields for blind_exit.

- Default route advertisement: daemon ensures blind_exit advertises the default route CIDR when missing, persisting state if it had to correct the posture.

Evidence: daemon.rs:930-1004 defines NodeRole, blind_exit command permissions, and route sanitization. daemon.rs:5489-5585 applies blind_exit dataplane behavior. daemon.rs:6709-6751 enforces blind_exit invariants and assignment validation. README.md:294-303 documents role semantics and final-hop orientation. start.sh:291-321, 357-365, 3413-3418, 3652-3861 show the current setup/menu enforcement points.

## 4.2 Linux dataplane and WireGuard backend

- Backend API shape: TunnelBackend exposes start, peer configuration, route application, exit mode, stats, and authoritative shared-transport methods.

- Kernel WireGuard path: linux_command configures a kernel WireGuard interface with ip link add type wireguard, wg set private-key/listen-port, ip address add, and ip link set up.

- Route and exit mode handling: linux_command reconciles routes and manages full-tunnel ip rule state for exit mode.

- Command-only transport caveat: the kernel WireGuard command backend reports an explicit blocker because it does not own authoritative packet I/O or the UDP socket. This matters for STUN/relay/traversal correctness, not for basic reachable-endpoint exit-node NAT.

- Userspace shared substrate: LinuxUserspaceSharedBackend exists and starts a TUN device, authoritative UDP socket, and userspace engine; it exposes authoritative transport identity/round-trip/send. This is relevant to the future plug-and-play transport path.

Evidence: rustynet-backend-api/src/lib.rs:52-62 defines ExitMode and RouteKind; 155-186 defines the backend trait. linux_command.rs:138-192 configures the kernel interface; 207-299 applies route reconciliation and exit tables; 444-466 implements set_exit_mode and the command-only blocker. userspace_shared/mod.rs:134-160 starts TUN/socket/engine runtime; 359-493 implements backend capabilities, route/exit application, and authoritative transport methods.

## 4.3 Systemd runtime and key custody hardening

- Systemd daemon: rustynetd.service waits for network-online, requires rustynetd-privileged-helper, loads environment from /etc/default/rustynetd, uses encrypted systemd credentials for WireGuard key passphrase material, and defaults to backend linux-wireguard.

- State paths: The service uses /run/rustynet, /var/lib/rustynet, and /etc/rustynet as runtime/state/config roots. These map well to a final appliance split between read-only rootfs and narrow writable state.

- Least privilege: The daemon runs as rustynetd, has NoNewPrivileges, ProtectSystem=strict, ProtectHome=true, empty CapabilityBoundingSet, and DeviceAllow only for /dev/net/tun.

- Privileged helper: The helper runs separately as root with bounded capabilities for network operations. The helper validates allowed programs and arguments for ip, nft, wg, sysctl, and related commands.

- Credential posture: The hardening documentation explicitly rejects persistent plaintext WireGuard passphrase files in the hardened Linux runtime path and requires encrypted systemd credential custody.

Evidence: scripts/systemd/rustynetd.service:1-62 defines startup dependencies, environment, backend/interface/listen-port/egress defaults, and encrypted credential loading; 63-95 defines restart and hardening controls. scripts/systemd/rustynetd-privileged-helper.service:10-45 defines helper execution and capabilities. RustynetdServiceHardening.md:1-8 and 28-114 document credential-only custody and service hardening. privileged_helper.rs:60-126 restricts allowed programs; 1133-1325 validates nft rules; 1475-1485 validates sysctl forwarding/IPv6 toggles.

## 4.4 Security and release-bar requirements already documented in the repo

- Crypto posture: production tunnel encryption is WireGuard-style authenticated encryption; no custom cryptographic protocol design is permitted in production paths.

- Signed control state: clients validate signed peer/control data before applying it; trusted authorization/signing state must fail closed when missing or corrupt.

- Default deny: default-deny ACL behavior is required across mesh, routes, and exit-node access.

- Leak prevention: tunnel and DNS fail-close behavior are required, and traversal transitions must be authenticated, replay-protected, and freshness-bounded.

- Performance gates: the repo defines Raspberry Pi-class budgets: idle daemon CPU <=2 percent of one core, idle RSS <=120 MB, reconnect target <=5 s, route/policy apply <=2 s p95, <=15 percent overhead vs baseline WireGuard path, and at least 24 h release-candidate soak.

Evidence: SecurityMinimumBar.md:13-33 defines crypto, signed control data, enrollment, and secret handling controls. SecurityMinimumBar.md:41-57 defines default-deny, privilege, and leak-prevention controls. SecurityMinimumBar.md:80-89 defines performance and soak minimums. dataplane.rs:1079-1105 contains a fail-close test for tunnel and DNS protected modes.

# 5. Software gaps for the sealed PCB appliance

| Gap | Repo evidence / reason | Required work for blind-exit PCB |
| --- | --- | --- |
| Claim-only appliance provisioning | Current README/start.sh describe first-run wizard, role selection, setup menus, and returning to a terminal control menu after setup. Static search found no dedicated rustynet-applianced or blind-exit claim-only service. | Add an appliance state machine: unclaimed -> provisioning -> claimed/operational -> factory-reset. Accept exactly one signed enrollment bundle for blind_exit and then disable provisioning. |
| No permanent operator surface | start.sh still implements admin/client/blind_exit role handling, menu behavior, exit selection, LAN toggles, and role confirmations. | Final image must not ship the general menu as the user surface. The appliance can keep internal diagnostics but should expose no shell, package manager, or permanent UI. |
| Wi-Fi uplink provisioning | PCB docs require onboard Wi-Fi, but the RustyNet repo does not contain a dedicated headless appliance Wi-Fi provisioning flow in the inspected files. | Add a minimal uplink provisioner for SSID/password/regulatory region. Provisioning should be temporary and physically gated or unclaimed-only. |
| Immutable OS image | Repo contains Debian/systemd-oriented services and release gates, but no Buildroot/read-only rootfs/A-B updater implementation was found in the inspected files. | Create final firmware image with read-only rootfs, signed A/B updates, writable state partition, watchdog, rollback, and factory reset. |
| Commit-bound build evidence | Uploaded zips do not include .git metadata; release-gate docs require git_commit matching HEAD. | Future builds must be generated from a git checkout with preserved commit identity and measured evidence artifacts. |
| Universal plug-and-forget reachability | Traversal docs say direct UDP is necessary but not sufficient and relay fallback is mandatory for robust plug-and-play. They also list missing live relay/direct evidence. | For MVP, allow static endpoint/manual port-forward/NAT-PMP where available. For full product UX, complete or rely on RustyNet traversal/relay infrastructure outside this exit-only PCB. |

Evidence: README.md:235-270 and 292-303 describe current wizard/role/menu behavior. start.sh:291-321, 3413-3418, 3652-3861 show role setup and disabled blind_exit actions inside a broader interactive script. PlugAndPlayTraversalRelayDeltaPlan_2026-03-29.md:112-124 says relay fallback is mandatory for robust plug-and-play; 239-247 lists missing production pieces; 248-263 says current live evidence does not yet support a plug-and-play cross-network claim.

# 6. Hardware and PCB architecture

## 6.1 Current PCB baseline found in the repo

- SoC and board class: the PCB docs describe a first-spin bring-up/debug board around Allwinner H616, sized like a Pi Zero 2 W-class appliance.

- Existing locked specs: H616, 512 MB DDR3/DDR3L, 8 GB eMMC, microSD recovery/reprovisioning, 10/100 Ethernet via RJ45 MagJack, two USB-A host ports, USB-C 5 V input, basic integrated Wi-Fi, SPI TFT, and eight buttons.

- Reference approach: Orange Pi Zero2 H616 schematic patterns are the primary integration reference for H616, DDR, Wi-Fi over SDIO, boot straps, UART debug, SD/eMMC, and USB-C power.

- Prototype philosophy: the repo emphasizes test points, 0-ohm isolation links, UART console, boot/recovery strap access, and microSD recovery for first spin debugging.

Evidence: rustyjack-h616-pcb-architecture-v0.md:1-4 defines first-spin H616 bring-up/debug intent. Lines 10-23 define the product goal and baseline peripherals. Lines 24-33 define Orange Pi Zero2 and Raspberry Pi Zero 2 W reference roles. Lines 35-43 define the instrumented bring-up philosophy. Lines 47-58 list locked specs.

## 6.2 Blind-exit PCB target hardware

| Subsystem | Repository baseline | Blind-exit target | Rationale |
| --- | --- | --- | --- |
| SoC | Allwinner H616. | H616/H618-class or equivalent ARMv8 SoC capable of mainline-ish Linux, WireGuard, nftables, Ethernet, Wi-Fi, and eMMC. | H616 baseline is present; exit-node software needs reliable Linux networking rather than display/UI horsepower. |
| RAM | 512 MB DDR3/DDR3L locked in existing watchdog PCB docs. | 1 GB absolute minimum; 2 GB preferred for unattended appliance headroom. | SecurityMinimumBar idle RSS target is <=120 MB, but OS, services, updates, logs, Wi-Fi, and recovery need margin. 512 MB should only proceed after ARM soak proof. |
| Storage | 8 GB eMMC plus microSD recovery. | 8-16 GB eMMC primary. Internal or non-user-facing microSD/FEL recovery for manufacturing and recovery. | Repo already selects eMMC and microSD recovery; final product should avoid encouraging user modification through exposed removable media. |
| Ethernet | 10/100 RMII PHY and MagJack. | Prefer Gigabit Ethernet. 10/100 acceptable only if product explicitly caps aggregate egress below 100 Mbps. | Exit-node traffic is network-bound. Current 10/100 baseline is usable for low throughput but is a deliberate product cap. |
| Wi-Fi | Basic integrated Wi-Fi over SDIO; prefer certified module. | Wi-Fi station-mode uplink; temporary setup AP only during provisioning if needed; external antenna/u.FL option strongly preferred. | Stakeholder requires Ethernet or wireless uplink. PCB docs already prefer SDIO module over raw RF. |
| USB | Two USB-A host ports for accessories/USB Wi-Fi dongles. | Remove user-facing USB-A from product revision unless needed internally. Keep factory-only USB/FEL access if necessary. | Single-purpose sealed appliance should not invite user peripherals or modification. |
| UI | SPI TFT plus eight buttons. | Remove display and eight-button UI. Keep one status LED plus one recessed setup/factory-reset button. | Provisioning should be claim-only; permanent UI is unnecessary and increases BOM, enclosure, and attack surface. |
| Debug | UART0 header, reset, boot/FEL straps, rail test points, isolation links. | Keep test pads and factory pads. Do not expose developer headers in the enclosure. | Bring-up still needs instrumentation; product should hide debug access. |
| Power | USB-C 5 V input; simple sink; no PD required. | USB-C 5 V input, 2-3 A budget, input fuse/TVS, brownout-safe eMMC behavior. | Repo baseline already matches USB-C power. Appliance must survive unattended power cycles. |

Evidence: rustyjack-h616-pcb-architecture-v0.md:119-140 defines 512 MB DDR strategy; 144-171 defines eMMC and microSD recovery; 183-202 defines 10/100 Ethernet; 206-229 defines two USB-A host ports; 233-252 defines SDIO Wi-Fi module preference; 256-271 defines SPI TFT/buttons; 275-298 defines debug/test instrumentation; 302-318 defines manufacturing/DFM assumptions. KiCad deliverables guide:149-161 maps these subsystems into schematic sheets; 248-257 assumes 6-layer controlled-impedance work for DDR/USB/Ethernet.

## 6.3 Revised PCB block diagram

USB-C 5 V input

-> protection / fuse / power tree / watchdog-capable reset

-> ARM SoC (H616/H618-class)

-> DDR3/DDR3L/LPDDR memory: 1 GB min, 2 GB preferred

-> eMMC: 8-16 GB primary boot/rootfs/state image storage

-> internal recovery: microSD or FEL/factory pads

-> Ethernet MAC/PHY: preferably 1x Gigabit RJ45 MagJack

-> Wi-Fi module over SDIO/PCIe/USB, Linux-supported, station-mode uplink

-> UART/factory debug pads, not user exposed

-> one status LED

-> one recessed setup/factory reset button

PCB direction: The existing H616 PCB docs are still valuable for SoC integration, DDR, power, eMMC, microSD, Wi-Fi, Ethernet connector hygiene, and bring-up strategy. They should not be treated as the final blind-exit product spec because their UI, USB expansion, and 512 MB/10/100 choices were made for a different watchdog board.

## 6.4 Board bring-up requirements

- Document every power rail with expected voltage, sequencing, test point location, and maximum bring-up current.

- Keep UART0 TX/RX/GND factory pads reachable before first boot; product enclosure should not expose them to users.

- Keep a forced recovery path: microSD boot, FEL/USB recovery, or fixture pads capable of re-imaging eMMC.

- Keep isolation links or DNP options for Wi-Fi module, Ethernet PHY, eMMC clock, and major rails on early spins.

- Write a Linux bring-up checklist for eMMC, Ethernet link, Wi-Fi station mode, /dev/net/tun, nftables, sysctl forwarding, RustyNet interface creation, and sustained exit traffic.

- Run thermal tests under sustained NAT/WireGuard load before freezing the enclosure.

Evidence: rustyjack-h616-pcb-architecture-v0.md:275-298 requires UART console, boot strap access, rail test points, and isolation links. Lines 322-355 list early decisions and require each low-level reference doc to end with KiCad implementation, test/validation, and risk-register sections.

# 7. Firmware and operating system plan

## 7.1 Prototype OS versus final OS

| Phase | Recommended OS/runtime | Why |
| --- | --- | --- |
| Prototype on dev board | Debian/Armbian or similar minimal Linux with systemd. | The current repo already ships Linux systemd units, privileged helper, WireGuard tooling assumptions, nft/ip/sysctl helper validation, and Linux host-profile behavior. This is the fastest path to measure throughput and reliability. |
| First custom board bring-up | Minimal Debian-style image while hardware is unstable. | Early hardware debug benefits from shell access, package tools, logs, and standard diagnostics. This is not the final user-facing image. |
| Final blind-exit appliance | Custom minimal Linux image, likely Buildroot or equivalent. | The product requirement is sealed and single-purpose: no package manager, no permanent shell/admin surface, read-only rootfs, signed update path, and only narrow writable state. |

Evidence: scripts/systemd/rustynetd.service:1-95 confirms the current Linux/systemd runtime assumptions. RustynetdServiceHardening.md:13-27 documents the service files and direct Rust service execution. No Buildroot/read-only-rootfs/A-B updater implementation was found in the inspected RustyNet or PCB trees; this is a recommendation, not an existing repo feature.

## 7.2 Final image layout

Bootloader / firmware region

Kernel A

Rootfs A: read-only, signed

Kernel B

Rootfs B: read-only, signed

Writable state partition:

/var/lib/rustynet/appliance-state

/var/lib/rustynet/keys/*.enc

/var/lib/rustynet/rustynetd.assignment

/var/lib/rustynet/rustynetd.trust

/var/lib/rustynet/membership.*

Wi-Fi uplink credentials, if used

bounded log ring buffer

Recovery/factory partition or factory reflash path

- The root filesystem should be mounted read-only in normal operation.

- Only RustyNet identity, keys, membership/trust/assignment bundles, Wi-Fi credentials, update slot state, and bounded logs should be writable.

- The image should not include apt, compilers, headers, Python, general shell access, a web admin UI, or SSH enabled by default.

- Factory reset should wipe mesh membership, WireGuard/RustyNet private keys, assignment bundles, Wi-Fi credentials, and logs; it should not wipe firmware trust roots unless a deliberate manufacturing recovery path does so.

- A/B updates should verify signatures, boot the inactive slot, mark good only after RustyNet health checks, and roll back automatically on failure.

Evidence: RustynetdServiceHardening.md:99-114 lists required runtime files for the hardened Linux runtime. scripts/systemd/rustynetd.service:13-62 shows the exact env paths used by the daemon. SecurityMinimumBar.md:63-66 requires signed artifacts, SBOM retention, and staged release tracks for security-sensitive rollout paths.

# 8. Provisioning model: claimable, not configurable

## 8.1 Device states

UNCLAIMED

- no mesh membership

- provisioning endpoint active only on LAN or temporary setup Wi-Fi

- status LED slow blink

PROVISIONING

- receives signed claim/enrollment bundle

- stores local identity, public key, encrypted key material, mesh assignment, trust roots

- optionally stores Wi-Fi uplink credentials

- validates role is blind_exit only

CLAIMED / OPERATIONAL

- provisioning endpoint disabled

- rustynetd runs as blind_exit

- NAT/firewall active

- status LED solid when online

FACTORY RESET

- physical button gated

- wipes mesh identity/state and Wi-Fi credentials

- returns to UNCLAIMED

## 8.2 Claim flow

- Ethernet first: If Ethernet has DHCP, device advertises a local unclaimed-device discovery record and waits for an authorized RustyNet admin/client tool to claim it.

- QR fallback: A QR code on the case can encode device_id, device public key, and one-time claim secret. The claim secret must be consumed exactly once.

- Temporary Wi-Fi setup: If no Ethernet is used, the physical setup window can start a temporary setup SSID. The user provides Wi-Fi credentials plus signed RustyNet claim material. The temporary network shuts down after claim.

- Role enforcement: The device rejects any enrollment bundle that would create admin, client, relay, or upstream-exit behavior. Only blind_exit is accepted.

- Post-claim behavior: The provisioning service exits and remains disabled unless the physical setup/factory button opens a controlled window or factory reset wipes state.

Admin node / app / CLI                   Blind-exit appliance

|                                         |

| discover device or scan QR              |

|---------------------------------------->|

| request identity/capabilities           |

|---------------------------------------->|

| device_id + device_public_key           |

|<----------------------------------------|

| signed blind_exit enrollment bundle     |

|---------------------------------------->|

| validate role, trust, freshness          |

| store state + disable provisioning       |

| start rustynetd as blind_exit            |

|<----------------------------------------|

| claimed / operational                   |

Implementation status: This is a required new appliance layer. It is aligned with repo role and signed-state concepts, but it was not found as an implemented claim-only appliance service in the uploaded tree.

Evidence: README.md:235-270 shows existing first-run bootstrap and signed assignment/trust workflows. SecurityMinimumBar.md:22-33 requires hardened enrollment and secret handling. daemon.rs:930-1004 and 6709-6751 provide the blind_exit restrictions that the appliance provisioner must enforce. start.sh:319-321 mentions factory reset and fresh key material as the route to change blind_exit role, but no complete hardware appliance reset flow was found.

# 9. Networking and security policy for the appliance

## 9.1 Default dataplane policy

Allow:

- RustyNet tunnel traffic needed for authorized peers

- rustynet0 -> WAN internet egress

- WAN -> rustynet0 established/related return traffic

- NAT/masquerade from RustyNet mesh CIDR out selected WAN interface

Drop by default:

- inbound WAN management traffic

- RustyNet peer access to host LAN / private subnets unless signed policy permits it

- unknown local services

- DNS or tunnel leakage outside protected policy

- stale, unsigned, or malformed assignment/trust/traversal state

- WAN interface selection should prefer Ethernet when link is present and DHCP succeeds; Wi-Fi should be fallback or explicitly provisioned uplink.

- The systemd service already exposes RUSTYNET_EGRESS_INTERFACE=auto; the appliance should make this deterministic and observable, not user-configurable through a broad settings surface.

- The firewall generator should create policy for the selected WAN interface only and reapply if the uplink changes.

- LAN access should remain disabled for blind_exit unless a future signed policy explicitly enables and scopes it. The product default should be internet egress only.

- The device should never act as a general NAT router for arbitrary local clients; it routes RustyNet tunnel traffic only.

Evidence: Requirements.md:70-89 defines exit-node and LAN access requirements, including fail-close behavior. daemon.rs:5575-5580 disables LAN access and selected_exit_node for blind_exit. privileged_helper.rs:1133-1325 validates nft accept/drop/forward/masquerade schemas; 1475-1485 restricts sysctl forwarding toggles. scripts/systemd/rustynetd.service:50-52 defines egress auto and auto-port-forward defaults.

## 9.2 Reachability caveat

- The PCB being exit-only does not eliminate the network reachability problem. Other RustyNet nodes must still be able to reach this exit node through home NATs and firewalls.

- For MVP hardware validation, reachable endpoint, port forwarding, same-LAN testing, or opportunistic NAT-PMP/UPnP can be accepted as lab constraints.

- For a consumer plug-and-forget product, the broader RustyNet system still needs robust traversal and/or relay fallback. The relay role can live elsewhere; it does not need to be on this PCB.

- Do not represent the blind-exit PCB as universally plug-and-play until there is fresh, commit-bound live cross-network evidence.

Evidence: PlugAndPlayTraversalRelayDeltaPlan_2026-03-29.md:112-124 states direct UDP is necessary but insufficient and relay fallback is mandatory for robust plug-and-play. Lines 161-174 distinguish exit node and relay roles. Lines 239-247 list missing production traversal/relay pieces, and lines 248-263 state current evidence does not yet support plug-and-play cross-network claims.

# 10. Work plan to get the low-level ARM PCB ready

| Phase | Goal | Concrete tasks | Exit criteria |
| --- | --- | --- | --- |
| 0. Requirements freeze | Turn this report into a locked product brief. | Freeze user count, throughput target, Ethernet/Wi-Fi behavior, LAN isolation default, setup UX, physical button/LED behavior, and allowed support/debug paths. | A one-page product requirements spec exists and no PCB schematic work starts before it is approved. |
| 1. ARM software prototype | Prove blind_exit on an off-the-shelf ARM Linux board. | Build current RustyNet from a git checkout; install rustynetd.service; force node_role=blind_exit; run Ethernet egress; run nft/NAT; run 3-user and 8-user traffic tests; collect CPU/RSS/temperature/log evidence. | Commit-bound report: build SHA, OS image, kernel, hardware, throughput, RSS, CPU, temp, 24 h soak. |
| 2. Claim-only appliance mode | Remove operator setup from runtime path. | Add rustynet-applianced or rustynetd --appliance-mode. Implement unclaimed/claimed/factory-reset state, one-time claim bundle validation, role lock, and provisioning disable after claim. | Fresh install can be claimed once, becomes blind_exit, rejects all other roles/actions, survives reboot, and returns to unclaimed after factory reset. |
| 3. PCB spec revision | Convert H616 watchdog board into blind-exit appliance board. | Remove TFT/buttons/USB-A expansion; select RAM/storage/Ethernet/Wi-Fi targets; update schematic sheet list; add factory pads/status LED/setup button; define eMMC/recovery path; update BOM and DFM assumptions. | Revised architecture doc and schematic checklist approved before KiCad capture. |
| 4. First custom PCB bring-up | Boot Linux and validate all hardware blocks. | Power rails, UART boot log, eMMC boot, microSD/FEL recovery, Ethernet link, Wi-Fi station mode, /dev/net/tun, nft, sysctl forwarding, RustyNet interface, sustained NAT. | Bring-up log with pass/fail per subsystem, no brownout under load, and recoverable reflash path. |
| 5. Locked firmware image | Replace general Debian personality with sealed appliance image. | Build minimal image, read-only rootfs, writable state partition, encrypted key custody, no package manager/SSH/web UI, signed A/B updates, watchdog, rollback, log ring. | Power-loss and update-failure tests pass; rootfs remains read-only; provisioning and RustyNet runtime survive reboot. |
| 6. Release validation | Generate evidence that matches repo gates and product claims. | 24-72 h soak, 3/8 user concurrency, Ethernet/Wi-Fi uplink, factory reset, A/B rollback, corrupt-state fail-closed, port-forward/NAT traversal test matrix, thermal envelope. | Signed release candidate image, SBOM, commit-bound evidence, archive hashes, known-issue register, and manufacturing programming procedure. |

Evidence: SecurityMinimumBar.md:80-89 provides release-performance and soak targets. FreshInstallOSMatrixReleaseGate.md:17-85 defines measured, fresh, commit-bound gate behavior. rustyjack-h616-pcb-architecture-v0.md:337-355 requires low-level implementation docs with KiCad checklist, validation checklist, and risk register.

# 11. Validation matrix

| Area | Test | Pass condition |
| --- | --- | --- |
| Build provenance | Build from git checkout and record git rev-parse HEAD, archive hashes, toolchain, target triple, OS image, kernel, and board revision. | Evidence file contains exact 40-char commit and matches source checkout. |
| Boot and recovery | Cold boot, warm reboot, power pull during boot, power pull during write, forced recovery boot/reflash. | Device returns to operational or recovery state without corrupting immutable firmware slots. |
| Claim flow | Unclaimed Ethernet claim, optional Wi-Fi setup claim, invalid role bundle, stale bundle, replayed claim, factory reset then re-claim. | Only valid blind_exit claim succeeds; provisioning disabled after claim; invalid/replayed claims fail closed. |
| Dataplane | Authorized peers route traffic out through device. Unauthorized peers, malformed state, stale assignment, missing key custody, and missing helper fail closed. | No unauthorized egress. Valid egress works. Failure modes are safe and observable. |
| LAN isolation | Attempt to reach host LAN/router/printer/private RFC1918 subnets from RustyNet clients. | Blocked by default unless an explicit signed LAN policy exists for a test case. |
| Throughput | Sustained aggregate egress at 50 Mbps minimum and 100 Mbps preferred target, with 3 normal and 8 peak simulated users. | No service crash, acceptable latency, no thermal throttling that violates product target. |
| Resource use | Measure idle and load CPU/RSS on target board. | Idle aligns with repo performance budget or any variance is justified by appliance services. |
| Wi-Fi uplink | Provision Wi-Fi, route RustyNet traffic, reboot, wrong password, AP unavailable, reconnect after AP returns. | Correct credentials persist; no credentials leak; reconnect works; wired preference behavior is deterministic. |
| Firewall/NAT | Inspect nft rules and sysctl state before/after RustyNet starts and after failures. | Rules match generated policy; forwarding/NAT only where expected; fail-closed drops exist. |
| Update/rollback | Apply good image, bad signature, bad health image, power loss during update. | Only signed updates apply; bad update rolls back; no bricked normal path. |
| Soak | 24 h minimum release-candidate soak; preferably 72 h for the appliance PCB. | No memory leak trend, no thermal instability, no filesystem growth beyond bounded log policy, no connectivity drift. |

# 12. Key risks, decisions, and recommendations

## 12.1 Decisions to lock before schematic capture

- Throughput target: choose whether product is capped around 50 Mbps, targets 100 Mbps, or needs more. This decides Ethernet and thermal priorities.

- RAM target: either prove 512 MB under soak or move baseline to 1-2 GB. Recommendation: 2 GB for unattended reliability unless BOM pressure is extreme and benchmarks justify less.

- Ethernet target: either accept 10/100 as a deliberate cap or revise to Gigabit. Recommendation: Gigabit if the SoC/PHY/BOM permits it.

- Wi-Fi role: station-mode uplink plus temporary provisioning AP only. Do not make the device a permanent access point/router unless requirements change.

- Recovery exposure: keep microSD/FEL available for factory/engineering; decide whether user enclosure exposes it. Recommendation: do not expose removable boot media to normal users.

- Secure boot/trust root: decide based on selected SoC support. At minimum, use signed update images; for stronger unmodifiable behavior, use verified boot if feasible.

## 12.2 Highest risks

| Risk | Why it matters | Mitigation |
| --- | --- | --- |
| No commit metadata in current archives | Release and build claims cannot be tied to exact source revision. | Use git checkout for all future builds and evidence. Record commit in image metadata and report artifacts. |
| Appliance provisioning absent | User cannot safely enroll device without exposing a broader admin/menu surface. | Implement claim-only state machine before final PCB firmware decisions. |
| 512 MB RAM baseline may be too tight | Unattended Linux appliance needs headroom for RustyNet, WireGuard, nft, Wi-Fi, updates, logs, and recovery. | Benchmark 512 MB on prototype. Prefer 2 GB if product cost allows. |
| 10/100 Ethernet baseline caps product | Exit nodes are network-throughput devices. 10/100 may be acceptable but must be a conscious cap. | Test H616/H618 board throughput; revise Ethernet to Gigabit if target exceeds low-cost 10/100 class. |
| Wi-Fi driver/module fragility | A plug-and-forget device can fail if Wi-Fi is unstable or needs unsupported firmware. | Choose a Linux-supported module with stable firmware, external antenna option, and soak-test reconnect behavior. |
| Plug-and-play reachability not complete | Home NATs may prevent remote peers from reaching exit node without traversal/relay. | Scope MVP to reachable endpoints; complete broader RustyNet traversal/relay infrastructure before consumer plug-and-play claims. |
| Immutable image not implemented | A Debian host remains modifiable and broad-purpose. | Build final image with read-only rootfs, no package manager, A/B signed updates, watchdog, and factory reset. |

# 13. Source evidence index

| Claim area | Primary source references |
| --- | --- |
| Workspace and toolchain | Cargo.toml:1-17, 20-30; rust-toolchain.toml:1-4; crates/rustynet-backend-wireguard/Cargo.toml:8-13; third_party/boringtun/Cargo.toml:1-16; third_party/rustynet-tun/Cargo.toml:1-9. |
| Current wizard/operator model | README.md:235-270, 292-303; start.sh:291-321, 357-365, 3413-3418, 3652-3861. |
| blind_exit role and restrictions | README.md:294-303; crates/rustynetd/src/daemon.rs:930-1004, 5489-5585, 6709-6751, tests around 20033-20213. |
| Backend/dataplane | crates/rustynet-backend-api/src/lib.rs:52-62, 87-93, 155-186; crates/rustynet-backend-wireguard/src/linux_command.rs:138-192, 207-299, 444-466; crates/rustynet-backend-wireguard/src/userspace_shared/mod.rs:134-160, 359-493; crates/rustynetd/src/dataplane.rs:392-470, 1079-1105. |
| Systemd hardening and key custody | scripts/systemd/rustynetd.service:1-95; scripts/systemd/rustynetd-privileged-helper.service:10-45; documents/operations/RustynetdServiceHardening.md:1-8, 28-114. |
| Privileged helper and firewall command policy | crates/rustynetd/src/privileged_helper.rs:60-126, 900-929, 1133-1325, 1475-1485. |
| Security and performance gates | documents/SecurityMinimumBar.md:13-33, 41-57, 63-66, 80-89; documents/operations/FreshInstallOSMatrixReleaseGate.md:17-85. |
| Platform support truth | documents/operations/PlatformSupportMatrix.md:76-106. |
| Traversal/reachability caveat | documents/operations/active/PlugAndPlayTraversalRelayDeltaPlan_2026-03-29.md:112-124, 144-174, 206-263; documents/operations/active/ProductionTransportOwningWireGuardBackendPlan_2026-03-31.md:25-62, 97-110, 183-219. |
| Current PCB architecture | rustyjack-h616-pcb-architecture-v0.md:1-58, 69-83, 119-171, 183-202, 206-252, 256-318, 322-355, 359-370; kicad-h616-deliverables-guide.md:149-161, 248-257, 349-401. |

# 14. Final blind-exit PCB checklist

## Hardware checklist

- ARMv8 Linux-capable SoC selected with stable DDR/eMMC/Ethernet/Wi-Fi reference path.

- 1 GB RAM minimum proven or 2 GB selected for headroom.

- 8-16 GB eMMC, internal recovery path, no exposed user boot media unless deliberately accepted.

- One RJ45 Ethernet, preferably Gigabit; Ethernet-first uplink policy documented.

- Wi-Fi module with stable Linux support, station mode, temporary setup mode, and antenna plan.

- USB-C 5 V input with protection and brownout behavior validated.

- Status LED and recessed setup/factory-reset button only.

- Factory UART/FEL/test pads present but not user exposed.

- No permanent display, no eight-button UI, no user GPIO header, no general USB expansion.

## Software checklist

- rustynetd runs only as blind_exit on the appliance.

- Provisioning service implements unclaimed/claimed/factory-reset state machine.

- Only signed blind_exit enrollment bundle is accepted.

- Provisioning endpoint is disabled after claim.

- Root filesystem is read-only in normal operation.

- No package manager, compiler, SSH-by-default, or permanent web UI.

- Writable state limited to RustyNet identity, encrypted keys, signed bundles, Wi-Fi credentials, update state, and bounded logs.

- A/B signed update and rollback path implemented.

- Factory reset wipes mesh/Wi-Fi/log state and returns to unclaimed.

- Firewall blocks LAN/private-subnet reachability by default and NATs RustyNet traffic only out selected WAN.

## Evidence checklist

- Build report includes exact git commit, source hashes, toolchain, target, OS image, and board revision.

- Fresh ARM build and tests run on the actual target board.

- 24 h minimum soak passes; 72 h preferred before hardware freeze.

- 3-user normal and 8-user peak traffic scenarios pass.

- Ethernet and Wi-Fi uplink behavior are both tested.

- Power-loss, corrupt-state, missing-key, stale-assignment, and update-failure cases fail closed.

- Thermal, memory, log growth, and eMMC write behavior are measured.

- Manufacturing procedure covers first boot, key generation/provisioning, image flashing, test fixture, and final seal.

# 15. Conclusion

The low-level blind-exit PCB is feasible, but the first deliverable should be a sealed appliance software slice rather than a custom board. The RustyNet repo already contains much of the exit-node dataplane and role-enforcement foundation. The custom PCB should not be designed until the claim-only blind_exit appliance mode, deterministic WAN selection, default LAN isolation, and measured ARM throughput/soak evidence exist.

The current H616 PCB archive is a useful bring-up reference, not the final product spec. Its SoC, eMMC, microSD recovery, USB-C power, Wi-Fi-module, UART, test-point, and DFM thinking are valuable. Its 512 MB RAM, 10/100 Ethernet, display, buttons, USB expansion, and developer-board affordances should be revised for a sealed RustyNet exit appliance.

The next practical step is to create a commit-bound ARM prototype build. That build should run blind_exit on a cheap ARM Linux board, collect throughput/resource/soak data, and prove the one-time claim/enrollment UX. Once that passes, the PCB spec can be frozen with fewer unknowns and a much lower risk of building the wrong board.

Evidence: This conclusion combines repo-confirmed blind_exit/software foundations with the PCB baseline deltas documented above. The concrete repo anchors are README.md:294-303, daemon.rs:930-1004 and 6709-6751, scripts/systemd/rustynetd.service:1-95, SecurityMinimumBar.md:80-89, FreshInstallOSMatrixReleaseGate.md:17-85, and rustyjack-h616-pcb-architecture-v0.md:1-58 and 183-252.